EXPERIMENT 7

AIM: Implementation of wifi channel in a network using ns3

NS-3 CODE:

#include "ns3/core-module.h"

#include "ns3/point-to-point-module.h"

#include "ns3/network-module.h"

#include "ns3/applications-module.h"

#include "ns3/wifi-module.h"

#include "ns3/mobility-module.h"

#include "ns3/csma-module.h"

#include "ns3/internet-module.h"

// Default Network Topology

//

//   Wifi 10.1.3.0

//                 AP

//  *    *    *    *

//  |    |    |    |    10.1.1.0

// n5   n6   n7   n0 -------------- n1   n2   n3   n4

//                   point-to-point  |    |    |    |

//                                   ================

//                                     LAN 10.1.2.0

using namespace ns3;

NS_LOG_COMPONENT_DEFINE("ThirdScriptExample");

int main(int argc, char* argv[])

{

bool verbose = true;

uint32_t nCsma = 3;

uint32_t nWifi = 3;

bool tracing = false;

CommandLine cmd;

cmd.AddValue("nCsma", "Number of \"extra\" CSMA nodes/devices", nCsma);

cmd.AddValue("nWifi", "Number of wifi STA devices", nWifi);

cmd.AddValue("verbose", "Tell echo applications to log if true", verbose);

cmd.AddValue("tracing", "Enable pcap tracing", tracing);

cmd.Parse(argc, argv);

// The underlying restriction of 18 is due to the grid position

// allocator's configuration; the grid layout will exceed the

// bounding box if more than 18 nodes are provided.

if (nWifi > 18) {

std::cout << "nWifi should be 18 or less; otherwise grid layout exceeds the bounding box" << std::endl;

return 1;

}

if (verbose) {

LogComponentEnable("UdpEchoClientApplication", LOG_LEVEL_INFO);

LogComponentEnable("UdpEchoServerApplication", LOG_LEVEL_INFO);

}

NodeContainer p2pNodes;

p2pNodes.Create(2);

PointToPointHelper pointToPoint;

pointToPoint.SetDeviceAttribute("DataRate", StringValue("5Mbps"));

pointToPoint.SetChannelAttribute("Delay", StringValue("2ms"));

NetDeviceContainer p2pDevices;

p2pDevices = pointToPoint.Install(p2pNodes);

NodeContainer csmaNodes;

csmaNodes.Add(p2pNodes.Get(1));

csmaNodes.Create(nCsma);

CsmaHelper csma;

csma.SetChannelAttribute("DataRate", StringValue("100Mbps"));

csma.SetChannelAttribute("Delay", TimeValue(NanoSeconds(6560)));

NetDeviceContainer csmaDevices;

csmaDevices = csma.Install(csmaNodes);

NodeContainer wifiStaNodes;

wifiStaNodes.Create(nWifi);

NodeContainer wifiApNode = p2pNodes.Get(0);

YansWifiChannelHelper channel = YansWifiChannelHelper::Default();

YansWifiPhyHelper phy = YansWifiPhyHelper::Default();

phy.SetChannel(channel.Create());

WifiHelper wifi;

wifi.SetRemoteStationManager("ns3::AarfWifiManager");

WifiMacHelper mac;

Ssid ssid = Ssid("ns-3-ssid");

mac.SetType("ns3::StaWifiMac",

"Ssid", SsidValue(ssid),

"ActiveProbing", BooleanValue(false));

NetDeviceContainer staDevices;

staDevices = wifi.Install(phy, mac, wifiStaNodes);

mac.SetType("ns3::ApWifiMac",

"Ssid", SsidValue(ssid));

NetDeviceContainer apDevices;

apDevices = wifi.Install(phy, mac, wifiApNode);

MobilityHelper mobility;

mobility.SetPositionAllocator("ns3::GridPositionAllocator",

"MinX", DoubleValue(0.0),

"MinY", DoubleValue(0.0),

"DeltaX", DoubleValue(5.0),

"DeltaY", DoubleValue(10.0),

"GridWidth", UintegerValue(3),

"LayoutType", StringValue("RowFirst"));

mobility.SetMobilityModel("ns3::RandomWalk2dMobilityModel",

"Bounds", RectangleValue(Rectangle(-50, 50, -50, 50)));

mobility.Install(wifiStaNodes);

mobility.SetMobilityModel("ns3::ConstantPositionMobilityModel");

mobility.Install(wifiApNode);

InternetStackHelper stack;

stack.Install(csmaNodes);

stack.Install(wifiApNode);

stack.Install(wifiStaNodes);

Ipv4AddressHelper address;

address.SetBase("10.1.1.0", "255.255.255.0");

Ipv4InterfaceContainer p2pInterfaces;

p2pInterfaces = address.Assign(p2pDevices);

address.SetBase("10.1.2.0", "255.255.255.0");

Ipv4InterfaceContainer csmaInterfaces;

csmaInterfaces = address.Assign(csmaDevices);

address.SetBase("10.1.3.0", "255.255.255.0");

address.Assign(staDevices);

address.Assign(apDevices);

UdpEchoServerHelper echoServer(9);

ApplicationContainer serverApps = echoServer.Install(csmaNodes.Get(nCsma));

serverApps.Start(Seconds(1.0));

serverApps.Stop(Seconds(10.0));

UdpEchoClientHelper echoClient(csmaInterfaces.GetAddress(nCsma), 9);

echoClient.SetAttribute("MaxPackets", UintegerValue(1));

echoClient.SetAttribute("Interval", TimeValue(Seconds(1.0)));

echoClient.SetAttribute("PacketSize", UintegerValue(1024));

ApplicationContainer clientApps = echoClient.Install(wifiStaNodes.Get(nWifi - 1));

clientApps.Start(Seconds(2.0));

clientApps.Stop(Seconds(10.0));

Ipv4GlobalRoutingHelper::PopulateRoutingTables();

Simulator::Stop(Seconds(10.0));

if (tracing == true) {

pointToPoint.EnablePcapAll("third");

phy.EnablePcap("third", apDevices.Get(0));

csma.EnablePcap("third", csmaDevices.Get(0), true);

}

Simulator::Run();

Simulator::Destroy();

return 0;

}

OUTPUT

>> ./waf --run scratch/third
Waf: Entering directory `/home/pc-120/Desktop/ns-allinone-3.28/ns-3.28/build'
Waf: Leaving directory `/home/pc-120/Desktop/ns-allinone-3.28/ns-3.28/build'
Build commands will be stored in build/compile_commands.json
'build' finished successfully (0.503s)
At time 2s client sent 1024 bytes to 10.1.2.4 port 9
At time 2.01794s server received 1024 bytes from 10.1.3.3 port 49153
At time 2.01794s server sent 1024 bytes to 10.1.3.3 port 49153
At time 2.03371s client received 1024 bytes from 10.1.2.4 port 9