EXPERIMENT 3

AIM: Create a topology of two nodes separated by point to point link network separated by a  delay. Deploy UDP Echo application by transferring packets at an interval of 1 second.

NS-3 CODE:

#include "ns3/core-module.h"

#include "ns3/network-module.h"

#include "ns3/internet-module.h"

#include "ns3/point-to-point-module.h"

#include "ns3/applications-module.h"

using namespace ns3;

// declares a logging component called FirstScriptExample that allows you to enable // and disable console message logging

NS_LOG_COMPONENT_DEFINE("FirstScriptExample");

int main(int argc, char* argv[])

{

CommandLine cmd;

cmd.Parse(argc, argv);

Time::SetResolution(Time::NS);
// enable two logging components inbuilt into EchoClient and EchoServer

// applications

LogComponentEnable("UdpEchoClientApplication", LOG_LEVEL_INFO);

LogComponentEnable("UdpEchoServerApplication", LOG_LEVEL_INFO);

// create the ns-3 Node objects that will represent the computers

NodeContainer nodes;

nodes.Create(2);

// instantiates a PointToPointHelper object on the stack and set its attributes

PointToPointHelper pointToPoint;

pointToPoint.SetDeviceAttribute("DataRate", StringValue("5Mbps"));

pointToPoint.SetChannelAttribute("Delay", StringValue("2ms"));

// use a NetDeviceContainer to hold the list of all NetDevice objects

NetDeviceContainer devices; // like a Network Interface Card

devices = pointToPoint.Install(nodes);

// install an Internet Stack on each of the nodes in the node container.

InternetStackHelper stack;

stack.Install(nodes);

// declare an address helper object to allocate IPs from network 10.1.1.0 using               // the mask 255.255.255.0 to define the allocatable bits

Ipv4AddressHelper address;

address.SetBase("10.1.1.0", "255.255.255.0");

// performs the actual address assignment

Ipv4InterfaceContainer interfaces = address.Assign(devices);

// set up a UDP echo server application on one of the nodes

UdpEchoServerHelper echoServer(9);

ApplicationContainer serverApps = echoServer.Install(nodes.Get(1));

serverApps.Start(Seconds(1.0)); // start server for 1 sec to 10 sec.

serverApps.Stop(Seconds(10.0));

// set up UDP echo client application on port 9 and set its attributes

UdpEchoClientHelper echoClient(interfaces.GetAddress(1), 9);

echoClient.SetAttribute("MaxPackets", UintegerValue(1));

echoClient.SetAttribute("Interval", TimeValue(Seconds(1.0)));

echoClient.SetAttribute("PacketSize", UintegerValue(1024));

// install applications on node 1

ApplicationContainer clientApps = echoClient.Install(nodes.Get(0));

clientApps.Start(Seconds(2.0));

clientApps.Stop(Seconds(10.0));

// actually run the simulation, then clean up with the destructor

Simulator::Run();

Simulator::Destroy();

return 0;

}